- Given a real-world scene captured under low light, describe how the image sensing and acquisition process influences the final digital image.

Answer: Image Sensing and Acquisition Under Low-Light Conditions

When a real-world scene is captured under low-light conditions, the image sensing and acquisition process has a significant impact on the quality of the final digital image.

Light from the Scene

The objects in the scene reflect only a small amount of light because the environment is dark.

This limited light enters the camera through the lens.

Image Sensing

The camera's image sensor (such as a CCD or CMOS sensor) converts the incoming light into electrical signals.

In low light, fewer photons reach the sensor, resulting in weaker electrical signals.

Image Acquisition

The analog electrical signals are converted into digital values using an Analog-to-Digital Converter (ADC).

Each pixel is assigned an intensity value, forming the digital image.

Effects on the Final Digital Image

Increased Noise: Weak signals make the image more susceptible to sensor noise, producing grainy or speckled images.

Low Brightness: The image appears dark because insufficient light is captured.

Loss of Detail: Fine textures and edges become difficult to distinguish.

Reduced Contrast: Differences between bright and dark areas become less noticeable.

Color Distortion: Colors may appear faded or inaccurate due to poor light.

Improvement Techniques

Increase the camera's exposure time to collect more light.

Use a larger aperture (lower f-number) to allow more light into the camera.

Increase the ISO sensitivity, though this also increases noise.

Apply image enhancement techniques such as denoising, histogram equalization, or brightness adjustment after acquisition.

Conclusion

The image sensing and acquisition process determines how effectively a camera captures and converts light into a digital image. Under low-light conditions, insufficient light reaching the sensor leads to darker images, increased noise, reduced contrast, and loss of detail. Proper camera settings and image enhancement techniques can improve the quality of the final digital image.