10)An image captured with low resolution fails to detect objects clearly. Explain how sampling and image formation affect object visibility.

Answer:

Effect of Sampling on Object Visibility

Sampling determines the spatial resolution of a digital image by deciding how many pixels are used to represent the captured scene. When an image is captured with low resolution, the sampling rate is too low, causing small objects to occupy only a few pixels. As a result, important details such as edges, shapes, and textures are lost, making the objects appear blurred or indistinct. If the sampling rate is below the required level (violating the Nyquist sampling criterion), aliasing occurs, where fine details are distorted or disappear completely. Therefore, inadequate sampling significantly reduces the visibility and detectability of objects in an image.

Effect of Image Formation on Object Visibility

Image formation refers to the process by which light from an object is focused onto the camera sensor to create an image. Factors such as poor focus, motion blur, low lighting, lens imperfections, and sensor noise can degrade image quality during this process. These factors spread the object's light across multiple pixels, reducing sharpness and contrast. In a low-resolution image, this degradation becomes even more severe because fewer pixels are available to represent the object accurately. Consequently, the object's features become difficult to distinguish, leading to poor visibility and reduced detection accuracy.

Conclusion

Both sampling and image formation play a crucial role in object visibility. Low sampling reduces spatial detail by using fewer pixels, while poor image formation introduces blur and noise that further degrade image quality. To improve object visibility, cameras should use higher spatial resolution, proper focus, sufficient lighting, and appropriate image enhancement techniques, ensuring that objects are captured clearly and can be detected accurately by Computer Vision systems.