4)Differentiate how pixel resolution and intensity resolution affect image representation in a practical application like medical imaging.

Solution

Pixel Resolution (Spatial Resolution):

Pixel resolution is determined by sampling and refers to the number of pixels used to represent an image.

A higher pixel resolution produces a sharper image with more visible details.

In medical imaging, high spatial resolution helps doctors identify small structures such as tiny tumors, fractures, or blood vessels.

If pixel resolution is low, the image becomes blurred and fine details may be lost.

Intensity Resolution (Radiometric Resolution):

Intensity resolution is determined by quantization and refers to the number of gray levels available in an image.

A higher intensity resolution provides smoother brightness transitions and better contrast.

Medical images such as X-rays, CT scans, and MRI scans commonly use 12-bit or 16-bit intensity resolution to distinguish tissues with very small density differences.

Lower intensity resolution (e.g., 8-bit) may make different tissues appear similar, reducing diagnostic accuracy.

Difference

| Pixel Resolution | Intensity Resolution |
| --- | --- |
| Determined by Sampling | Determined by Quantization |
| Controls image sharpness and detail | Controls brightness and contrast |
| Measured by number of pixels (e.g., 1024 × 1024) | Measured by bit depth (e.g., 8-bit, 12-bit, 16-bit) |
| Higher resolution reveals small objects | Higher resolution reveals subtle gray-level differences |
| Important for detecting tiny tumors and fractures | Important for differentiating tissues with similar densities |

Conclusion

In medical imaging, pixel resolution determines how clearly anatomical structures are visible, while intensity resolution determines how well different tissues can be distinguished based on their brightness. Both are essential for accurate diagnosis: high spatial resolution improves detail, and high intensity resolution improves tissue contrast.